IPv4 Packet Structure

The fields you should know

| Field | Purpose |
| --- | --- |
| Version | IP version (4 for IPv4). |
| IHL (Internet Header Length) | Size of the IPv4 header. Normally 20 bytes. |
| Total Length | Total size of the packet (header + data). |
| Identification | Identifies fragments of the same packet. |
| Flags | Controls fragmentation (e.g., Don't Fragment). |
| Fragment Offset | Position of a fragment in the original packet. |
| TTL (Time To Live) | Prevents packets from looping forever. Decreased by 1 at each router. |
| Protocol | Indicates the next-layer protocol (TCP, UDP, ICMP, etc.). |
| Header Checksum | Detects errors in the IPv4 header only. |
| Source IP Address | Sender's IP address. |
| Destination IP Address | Receiver's IP address. |
| Options | Optional extra information (rarely used). |

The most important fields for beginners

If you're studying networking or cybersecurity, these are the ones you should recognize immediately:

1. Source IP

Example:

192.168.1.10

The sender's IP address.

2. Destination IP

Example:

8.8.8.8

The destination IP address.

3. TTL (Time To Live)

Example:

TTL = 64

Every router decreases it by 1.

If it reaches 0:

➡️ The router drops the packet and usually sends back an ICMP Time Exceeded message.

4. Protocol

This tells IPv4 what's inside the payload.

| Value | Protocol |
| --- | --- |
| 1 | ICMP |
| 6 | TCP |
| 17 | UDP |

Example:

Protocol: TCP (6)

means the IPv4 payload contains a TCP segment.

5. Total Length

Indicates the total size of the IPv4 packet.

Example:

Total Length = 60 bytes

6. Header Checksum

Checks whether the IPv4 header was corrupted during transmission.

Important: It checks only the IPv4 header, not the payload.

Example in Wireshark

When you expand the Internet Protocol Version 4 section, you'll see something like:

Internet Protocol Version 4

Version: 4

Header Length: 20 bytes

Total Length: 60

Identification: 0x1234

TTL: 64

Protocol: TCP (6)

Header Checksum: 0xabcd

Source: 192.168.1.5

Destination: 10.10.41.192

You should be able to explain what each of these means.

DIFF BETWEEN FCS AND IPV4 CHECKSUM

Quick Comparison

| Feature | FCS | IPv4 Header Checksum |
| --- | --- | --- |
| Layer | Layer 2 (Data Link) | Layer 3 (Network) |
| Used in | Ethernet Frame | IPv4 Packet |
| Protects | Entire Ethernet frame (except the FCS field itself) | IPv4 header only |
| Algorithm | CRC (Cyclic Redundancy Check) | Internet Checksum (1's complement) |
| Stronger? | ✅ Yes | ❌ No |
| Recalculated by | Sender and receiver | Every router |

Where are they located?

Ethernet Frame

+--------------------------------------------------------------+

| Ethernet Header | IPv4 Packet | FCS (Trailer)                |

+--------------------------------------------------------------+

The FCS is the last field in the Ethernet frame.

IPv4 Packet

+------------------------------------------------------+

| IPv4 Header (Checksum) | TCP/UDP | Data              |

+------------------------------------------------------+

The IPv4 checksum is inside the IPv4 header.

What does each one check?

FCS

Checks:

- Ethernet Header ✅

- IPv4 Header ✅

- TCP/UDP Header ✅

- Application Data ✅

Basically, it checks almost the entire Ethernet frame.

IPv4 Header Checksum

Checks only:

- Version

- TTL

- Source IP

- Destination IP

- Protocol

- All other IPv4 header fields

It does NOT check:

- TCP data ❌

- UDP data ❌

- HTTP request ❌

- Payload ❌

Why do we need both?

Imagine you're sending an HTTP request.

HTTP GET

│

TCP

│

IPv4

│

Ethernet

If one bit changes during transmission...

FCS

Detects:

"The Ethernet frame was corrupted."

The frame is discarded.

IPv4 Checksum

Detects:

"The IPv4 header itself is corrupted."

This protects routing information like:

- Source IP

- Destination IP

- TTL

Why do routers recalculate the IPv4 checksum?

Every router decreases the TTL by 1.

Example:

Before router:

TTL = 64

Checksum = 0xABCD

After router:

TTL = 63

Since the IPv4 header changed, the old checksum is no longer valid.

The router computes a new IPv4 header checksum before forwarding the packet.

Does the router recalculate the FCS?

Yes, but for a different reason.

When a router receives an Ethernet frame:

- It checks the incoming FCS.

- Removes the old Ethernet header and trailer.

- Processes the IPv4 packet.

- Creates a new Ethernet frame for the next network hop.

- Calculates a new FCS for that new frame.

So the FCS changes because a new Ethernet frame is built, not just because the TTL changed.

Easy way to remember

Think of mailing a package.

📦 Ethernet Frame = Shipping box

📄 IPv4 Packet = Letter inside the box

- FCS checks whether the entire box arrived without damage.

- IPv4 Header Checksum checks whether the address written on the letter is still correct.